# AI Smart Glasses – Phase 0 Data Path Architecture Document

## Purpose of This Document

This document formally defines the data paths for Phase 0 (FPGA prototype) of the AI Smart Glasses program.
It clearly separates:

• Use Case • Full End‑to‑End Data Flow
• FPGA Components Involved
• RTL Components Required

This structure will be reused for ASIC Phase 1 and Phase 2.

# System-Level Architecture Overview

The Phase 0 FPGA prototype consists of four primary domains:

- Sensor Data Paths (Audio / IMU / Camera)

- DDR Memory Backbone

- NPU Compute Data Path

- Control & CPU Plane

DDR is treated as a first-class architectural block and not merely a memory element.

# USE CASE 0 – DDR Memory Backbone (System Data Movement)

## Purpose

Provide high-bandwidth external memory for:

• Model weights • Frame buffers • Audio buffers • Intermediate tensors • Embeddings

Without DDR, face recognition and CNN workloads cannot operate.

## Full Data Flow

External DDR
↕
DDR Controller
↕
AXI Interconnect
↕
DMA Engine
↕
Scratchpad SRAM
↕
NPU MAC Array

## FPGA Components Involved

• Zynq DDR Controller (PS side) • AXI High Performance Port • AXI Interconnect • AXI Master DMA • BRAM Scratchpad • NPU Core

## RTL Components Required

- AXI Master Interface
• Read channel
• Write channel
• Burst logic
• Address generator

- DMA Engine
• Transfer length register
• Base address register
• Stride logic (for convolution)
• Done interrupt logic

- Scratchpad Controller
• Dual-port SRAM interface
• Bank arbitration
• Read/write scheduler

- Memory Map Decoder
• Model base address
• Activation base address
• Frame buffer base address

# USE CASE 1 – Voice Phrase / Wake Word Detection

## Purpose

Detect trigger phrase locally with ultra-low latency and low power.

## Full Data Flow

Microphone
→ Audio ADC / Codec
→ I2S Interface
→ Audio Buffer (BRAM or DDR)
→ Feature Extraction (MFCC)
→ NPU (DS‑CNN inference)
→ INT32→INT8 Quantize
→ Softmax / Threshold
→ Interrupt to CPU

## FPGA Components Involved

• Microphone
• External Audio Codec
• I2S Controller
• Audio FIFO
• MFCC Processing Block
• NPU Core
• AXI Bus
• DDR
• ARM CPU
• Interrupt Controller

## RTL Components Required

- Audio Interface
• I2S RX Controller
• Audio FIFO
• Sample rate generator

- Audio Preprocessing
• Windowing block
• FFT engine
• Mel filterbank
• Log & normalization unit
• MFCC output formatter

- NPU Core (Reusable)
• MAC array
• Accumulator bank
• Scratchpad SRAM
• DMA engine
• Layer control FSM

- Post-Processing
• Quantization unit
• Softmax (optional)
• Threshold comparator
• Interrupt generator

# USE CASE 2 – Gesture Recognition (IMU-Based)

## Purpose

Detect predefined head or hand gestures using IMU data.

## Full Data Flow

IMU Sensor
→ I2C Interface
→ Sensor FIFO
→ Digital Filter
→ Feature Windowing
→ Tiny NN Model (NPU)
→ Classification Output
→ CPU Decision

## FPGA Components Involved

• IMU Sensor
• I2C Controller
• Sensor FIFO
• DSP Filter Block
• NPU Core
• AXI Bus
• CPU

## RTL Components Required

- Sensor Interface
• I2C Master Controller
• Sensor read FSM
• Data FIFO

- Signal Conditioning
• Low-pass filter
• Moving average block
• Feature packer

- Inference Engine
• Reused NPU core
• DDR model storage
• Scratchpad load logic

- Output Logic
• Class ID register
• Threshold comparator
• Interrupt logic

# USE CASE 3 – Face Recognition

## Purpose

Identify known person using camera input.

Two-stage pipeline: 1. Face Detection
2. Face Embedding + Similarity

## Full Data Flow

Camera
→ MIPI CSI‑2 Receiver
→ Frame DMA
→ DDR Frame Buffer
→ ISP-lite (resize + color conversion)
→ NPU (BlazeFace detection)
→ Bounding box output
→ Crop + Align
→ NPU (MobileFaceNet embedding)
→ Feature Vector
→ Cosine Similarity Engine
→ Identity Decision

## FPGA Components Involved

• Camera Module
• MIPI CSI‑2 RX
• Video DMA
• DDR
• Frame Buffer Controller
• ISP-lite Block
• NPU Core
• CPU

## RTL Components Required

- Camera Interface
• MIPI CSI‑2 receiver
• Pixel unpacker
• Frame sync logic
• Video DMA engine

- ISP-lite
• RGB converter
• Resize engine
• Line buffer
• Frame buffer controller

- NPU Core (Reusable)
• MAC array
• Scratchpad SRAM
• DMA engine
• Control FSM
• Quantization unit

- Post-Processing
• Bounding box parser
• Crop engine
• Vector normalization block
• Cosine similarity engine
• Threshold comparator

# Reusable Core Blocks Across All Use Cases

The following RTL blocks are common across all AI workloads:

• NPU MAC Array
• Accumulator Bank
• Scratchpad SRAM
• DMA Engine
• AXI Interconnect
• Quantization Unit
• Interrupt Controller
• CPU Register Interface

Pre-processing and post-processing differ per use case, but compute core remains identical.

# Architectural Conclusion

The system is organized into:

- IO Domain

- Memory Backbone Domain (DDR + DMA + Scratchpad)

- Compute Domain (NPU)

- Control Domain (CPU + Registers + Interrupts)

Proper DDR modeling and DMA scheduling are critical to avoid bandwidth bottlenecks.

This document forms the foundation for RTL implementation planning and future ASIC migration.